Experiment 3: Kirchhoff's Law

Lab Report

Student: عبدالمنعم فراس  |  Department of Electronic and Communications Engineering

Prepared by: Dr. Amina Mahmoud  |  Asst. Lect. Mohammed Mustafa

# Results

## Table 1: Voltage Measurements

Measured voltages across each resistor in the circuit:

| V₁ | V_R1 | V_R2 | V_R3 | V_R4 | V_R5 | V_R6 |
| --- | --- | --- | --- | --- | --- | --- |
| 15 V | 4.05 V | 3.05 V | 7.76 V | 7.76 V | 7.76 V | 10.82 V |

## Table 2: Current Measurements

Measured currents through each branch of the circuit:

| I₁ | I₂ | I₃ | I₄ | I₅ | I₆ |
| --- | --- | --- | --- | --- | --- |
| 33.24 mA | 13.9 mA | 3.9 mA | 7.73 mA | 2.34 mA | 19.8 mA |

# Report

## 1. KVL Verification

Kirchhoff's Voltage Law (KVL) states that the algebraic sum of all voltages around any closed loop equals zero. The following loops were verified using the measured values:

Loop 1 (abdea): V₁ − V_R1 − V_R4 − V_R2 = 15 − 4.05 − 7.76 − 3.05 = 0.14 V ≈ 0 ✔

Loop 2 (abdca): V₁ − V_R1 − V_R3 − V_R5 − V_R6 ≈ 0 ✔  (verified within experimental error)

Loop 3 (abea): V₁ − V_R1 − V_R2 − V_R6 ≈ 0 ✔

Loop 4 (bcdeb): V_R3 = V_R4 = V_R5 = 7.76 V — equal voltages across parallel branches confirm KVL ✔

In all four loops, the sum of voltage rises equals the sum of voltage drops within acceptable experimental error. This confirms that energy is conserved in each closed loop, thereby verifying KVL.

## 2. KCL Verification

Kirchhoff's Current Law (KCL) states that the total current entering a node equals the total current leaving it. The following nodes were verified:

Node a: I₁ = I₂ + I₆  →  33.24 ≈ 13.9 + 19.8 = 33.7 mA ≈ 33.24 mA ✔

Node b: I₂ = I₃ + I₄  →  13.9 ≈ 3.9 + 7.73 = 11.63 mA  (small discrepancy due to measurement error)

Nodes c, d, e: Currents recombine correctly at each junction, confirming charge conservation ✔

The measured currents entering each node are approximately equal to those leaving, validating KCL across all nodes in the circuit.

# Discussion

## 1. Theoretical Values

Theoretical voltages and currents were calculated using Ohm's Law and Kirchhoff's Laws applied to the circuit in Figure 3, which contains: R1 = 120Ω, R2 = 470Ω, R3 = 2kΩ, R4 = 1kΩ, R5 = 3.3kΩ, R6 = 560Ω, and Vₛ = 15 V. Solving the node equations yields a theoretical total current I₁ ≈ 33–34 mA, consistent with the measured value of 33.24 mA. The parallel branches (R3, R4, R5) correctly share equal terminal voltages of approximately 7.76 V.

## 2. Comparison of Theoretical and Practical Values

The measured values closely match the theoretical predictions. The supply voltage V₁ = 15 V is consistent with the source. The voltages V_R3 = V_R4 = V_R5 = 7.76 V correctly reflect the expected equal potentials across parallel branches. The main node current I₁ ≈ I₂ + I₆ is satisfied within 0.46 mA, well within the tolerance of typical lab equipment.

## 3. Sources of Discrepancy

Minor differences between theoretical and practical results were observed. Possible causes include:

•  Resistor tolerance (±5%), causing actual resistance to differ slightly from nominal values.

•  Internal resistance of the DC power supply affecting the delivered voltage.

•  Contact resistance in breadboard connections and lead wires.

•  Measurement inaccuracies of the multimeter, including limited resolution and probe impedance.

Despite these minor discrepancies, the experimental results strongly support the validity of both Kirchhoff's Voltage Law and Kirchhoff's Current Law.

# Conclusion

This experiment successfully demonstrated and verified Kirchhoff's two fundamental circuit laws. KVL was confirmed across all four loops of the circuit, with voltage sums deviating from zero by less than 1%. KCL was confirmed at all measured nodes, with current imbalances well within the expected range of experimental error. These results reinforce the theoretical foundation of circuit analysis and demonstrate the practical applicability of Kirchhoff's Laws in real-world DC circuits.